# Quick Start Guide

A short guide for the Metabolic MRI Processing Toolbox (MeMRI-PTB) which allows converting, processing, quantifying and visualizing raw-export metabolic imaging data i.e. multi-dimensional spectroscopy data from both Siemens and Philips MR platforms in MATLAB.

Designed for both beginner and experienced MATLAB users.

Contents

Background	3

Installation	4

Github in MATLAB	4

Custom root directory	4

Example data	5

CSIgui	5

Raw Data Converter	6

Vendor-specific Data Handling	6

Noise Source	7

How-To-Use	7

Philips	7

Siemens	7

Limitations	8

Process Data	9

How-To-Use	10

Input Arguments	10

Tutorial	11

Framework	12

MeMRI Data Format	12

Data-structure	12

Raw Data Converter	13

QOL Functionality	13

Processing	14

Visualization	14

Quantification	14

Alpha-release version does not include a quick-start guide.

# Background

Large x-nuclei spectroscopy data sets from high-field MR systems are becoming more prevalent in the recent decades due to technological advances in both hardware and computing power. Increases in RF coil density i.e. phased arrays, allow acquisition over large field of views (FOV) paving the way for upper and lower body MRS applications.

However, MR systems cannot handle these data sets using the conventional reconstruction pipelines and are often poorly documented. New processing methods have been introduced to increase sensitivity through new coil-combination and denoising methods. Therefor raw spectroscopy data is often exported for offline reconstruction.

Multi-centre studies with different MR-system vendors are a crucial part of translating academic research into clinical practice. Unfortunately, there is no consensus on processing methods nor their implementation. Therefore, we collaborated between 4 sites in 3 European countries to streamline data handling and visualization methodology. This resulted in this metabolic MR imaging processing toolbox available for the entire MR spectroscopy community.

Figure 1. Metabolic MRI Processing ToolBox application interface.

# Installation

MeMRI-PTB can be downloaded from github or retrieved via the MATLAB environment. See below for the latter assuming git is installed on your PC (see https://gitforwindows.org/ ).

Download:	https://github.com/Illuminate-MeMRI/Metabolic-MRI-Processing-Toolbox

## Github in MATLAB

Create an empty folder for MeMRI-PTB named Metabolic-MRI-Processing-Toolbox in MATLAB using the “Current Folder” pane via right-click > New > Folder. For a custom root directory name, see the paragraph below. Open the directory Metabolic-MRI-Processing-Toolbox, right-click inside the “Current Folder” pane and select “Source Control”. The window below will appear:

Figure 2. “Manage Files Using Source Control” window opened in MATLAB that allows setup and retrieval of a git-repository. Repository path shown is the path to use, sandbox path can be custom, see Custom root directory below.

Select Git as “Source Control Integration” and enter the github path “https://github.com/Illuminate-MeMRI/Metabolic-MRI-Processing-Toolbox” in the “Repository path” editfield. Click “retrieve” at the bottom right and wait until MATLAB has downloaded the required files.

### Custom root directory

A custom directory name for the github repository is possible but requires a change to the function memri_findRoot(). The defined root-directory is stored as variable “stroi” and must be changed to the desired directory name.

## Example data

The MRS example data is too large for default use with GitHub and therefore stored at a different (OneDrive-cloud) location.

Philips data 	[384MB] 	SHA256: 296359abc7478f3b43e1ec928850e3672f61b8b5c402885088c0f0ae86654242

Siemens data [307MB] 	SHA256: 88eac8a950a874b6a12d114cbc23663ce837c0c14d67f3068b881c5ef5dcfaa1

## CSIgui

The alpha release does not include a viewer, however is fully compatible with CSIgui. A CSIgui-lite will be released with MeMRI-PTB in a near future release.

GitHub: https://github.com/Sugocy/CSIgui

# Raw Data Converter

Script-of-interest:	memri_DataConvert_GUI()

Figure 3. Data converter for Philips and Siemens raw export metabolic MRI data. Input fields show an example for a Philips data set and includes; an image dicom, the raw spectroscopy file, the protocol parameters and a separate raw export noise measurement.

The processing application in MeMRI-PTB is MR platform-independent; therefore, a dedicated converter is used to convert raw-exported Philips and Siemens data into the MeMRI-PTB format in MATLAB, including the parameters required for subsequent processing and visualization. Noise and imaging data can be included but is optional. Data can be directly processed via the MeMRI-PTB application and/or stored to file.

Raw MRS data can be converted to a MATLAB-compatible structure using the MeMRI-PTB Data Converter, either through the GUI or memri_loadData(). The resulting structure contains raw data, indexing information, acquisition parameters, and data-source details.

## Vendor-specific Data Handling

Philips list/data exports are supported together with a protocol-information text file. If unavailable, the user is prompted for essential parameters: nucleus, field strength, receiver bandwidth, echo time and data domain.

Siemens Twix .dat exports contain the parameters required for processing.

For more details go, refer to the section Raw Data Converter in the Framework chapter.

## Noise Source

Separate noise measurements can be included during data conversion or imported later for supported vendors. Noise data from Philips’ list/data exports are used when available. As Siemens spectroscopy exports do not contain noise measurements, voxel-specific noise is generated when required.

## How-To-Use

Run the core-script “memri_DataConvert_GUI()” or run it via the processing application (see Processing Data,  “memri_ptb()”).  The GUI, the window in figure 3, should be visible.

### Philips

The list/data file must have equal filenames.

The protocol text-file can be omitted if it has the same name as the list/data files.

- Click “File..” to select a .data file. 	!! Combine Steps 1 & 2; multi-file select supported.

  - The first-column edit-field shows the path.

  - The second-column edit-field shows the filename.

  - The third-column drop-down menu will show “MRS data”.

- Repeat and select a protocol text-file.

  - The third-column drop-down menu will show “Protocol info”.

- Optional to add…

  - An enhanced DICOM file.

  - Separate noise .data file.

- Select “Save to File”

  - Save the converted data to mat-file.

  - The export file name will be equal to the source file.

- Select “Process via GUI”

  - Opens the MeMRI-PTB application and loads-in the data.

- Click Export

  - The “Vendor” text will change to “Philips” in red and will turn green when finished.

### Siemens

- Click “File..” to select a .dat file.

  - The first-column edit-field shows the path.

  - The second-column edit-field shows the filename.

  - The third-column drop-down menu will show “MRS data”.

- Optional to add…

  - A DICOM .ima-series.

  - Separate noise .dat file.

- Select “Save to File”

  - Save the converted data to mat-file.

  - The export file name will be equal to the source file.

- Select “Process via GUI”

  - Opens the MeMRI-PTB application and loads-in the data.

- Click Export

  - The “Vendor” text will change to “Siemens” in red and will turn green when finished.

### Limitations

The user is prompted for protocol information if the text-file is not found or supplied. Current Siemens DICOM implementation reads .IMA files only but will be updated in a future release.

For more details go, refer to the section Raw Data Converter in the Framework chapter.

# Process Data

The MeMRI-PTB framework is designed to keep processing tools independent of the MR system. Its layered architecture separates data management, core processing methodologies, and interfacing elements. Each processing method is implemented as a module, with new modules easily added by following a short guide. This modularity allows default processing pipelines to be extended or adapted with custom methods. Separating the data converter from the processing application also enables users to implement custom data-loading functions while retaining access to the included processing pipelines.

- Easy sharing of processing setups

  - Full logging implemented

  - Import/export of methods & parameters

- Easy reprocessing with new methods.

Modular Design

  - Easy addition of custom modules using provided guide

  - User-based custom code for unsupported file-formats

  - Enables custom processing pipelines.

Figure 4. Graphical user interface of the MeMRI-PTB application. Processing modules are loaded dynamically at runtime with integrated error checking. A live log window provides feedback during application use. Method-specific settings become available for adjustment upon selection, while the tab group also provides access to processing settings and loaded-data information.

## How-To-Use

Run “memri_ptb()” or “MeMRI_Processing_GUI()”, the latter is the application (.mlapp) and the primer a wrapper function to shorten the call.

- Load a mat-file with the memri-structure.

  - Path to file can entered in the edit-field.

  - Select a file via the file-button or menu bar.

- Select “Methods” and import to the “Operations” list.

  - The “default” button will load a default ordered list of operations.

- Check data parameters and details in the “Data” tab of the “Details” tab-group.

- Check parameter settings for the selected methods in the other tabs show in the  “Details” tab-group.

- Check processing settings in the “Processing” tab.

- Click “Start Processing”

  - Data will be saved according selected options

  - Log window will keep updating

  - Busy light turns green when finished.

- View will open the data in CSIgui if it is installed.

### Input Arguments

The application accepts input; either a filepath the a mat-file or the MeMRI-PTB structure. NB. It is advised to use memri_ptb(your_input) as MATLAB application will not update user-input after first initialisation i.e. if the application is already running. Please see memri_ptb() for detailed information.

# Tutorial

This tutorial will use the example data to demonstrate how to use the MeMRI-PTB. If example data is not present in the root-directory, please check Installation.

# Framework

The MeMRI-PTB framework is split into three hierarchical layers. The first layer comprises the core methodology, which is complemented by a second data management layer responsible for coordinating and overseeing its functionality. The final layer consists of application-level components for graphical interfacing, which provide access to and interaction with the underlying framework. Every processing method is a module in the MeMRI-PTB framework and allows custom processing methods (i.e. modules) to be added to the application or scripted processing pipelines easily with the provided guides.

This way, the MeMRI PTB combines four functionalities into one package:

- Conversion

- Processing

- Visualisation 		[TBD]

- Quantification 	[TBD]

The scripts-of-interest for each functionality plus a short description are given in the next paragraphs. These scripts-of-interest are found in the root-folder of MeMRI PTB.

## MeMRI Data Format

The MeMRI structure created by the converter and used by all other functions in the MeMRI-PTB has a few hard requirements and structure-fields. Table 1 shows an overview of the MeMRI data-structure including the description and required data-fields. Subfields are only described if required. Additional fields may be present in the NFO structure but not required for MeMRI-PTB to function.

### Data-structure

| Field name | Description |  | Required |
| --- | --- | --- | --- |
| .data | Multi-dimensional complex data array |  | ✓ |
| .labels | Cell list with a label for every dimension in “data”. |  | ✓ |
| .nfo | Structure with protocol parameters. See required sub-fields below. |  | ✓ |
|  | .nucleus | String for x-nucleus (‘31P’) |  |
|  | .fieldstrength | Magnetic field strength [T] |  |
|  | .bandwidth | Receiver bandwidth [Hz] |  |
|  | .TE | Echo time [s] |  |
|  | .domain | Current data domain (k-space = 0, time = 1, frequency = 2) |  |
| .noise | Structure for all noise data. Will use voxel-data to generate noise-components. |  | ~ |
|  | .data | Multi-dimensional complex data array |  |
|  | .labels | Cell list with a label for every dimension in “.noise.data”. |  |
|  | .domain | Current data domain (k-space = 0, time = 1, frequency = 2) |  |
| .images | MR images for localization |  |  |
| .source | Source file location and optionally the header of the source. |  |  |
| .filepath | File path of the memri-file. |  |  |
| .log | Includes all methods applied and is generated during processing. |  |  |
| .instance | Number given during processing to track log-data and files. Also used to store data with a unique identifier. Format: ”YYYY.MM.DD.HHMMSS” |  |  |

Table 1. Overview of the MeMRI-PTB data structure “memri” and the included field. Required fields are check marked in the left column, with optional fields, with additional information in the description, marked by a tilde. NB. The fieldnames shown are case sensitive.

## Raw Data Converter

Script(s) of interest: 	memri_DataConverter_Scripted.m

memri_DataConverter_GUI.m

Core file(s):		memri_loadData.m

Convert the raw Philips and Siemens MRS data into a MATLAB format that includes all necessary parameters to process the spectroscopy data (table 1). Philips exports raw MR spectroscopy data in a list- & data- file format, includes a noise measurement, but lacks protocol information and parameters. Raw-exports from Siemens systems are in a dat-file format and include all protocol parameters but does not include any noise measurements. The core script, memri_loadData(), supports both data formats and handles their loading, conversion to complex spectroscopy data, and export for further processing.

|  | Siemens | Philips |
| --- | --- | --- |
| File | *.dat | *.list / *.data |
| Data | IEEE-LE (uint32) | IEEE-LE (float32) |
| Noise | ✗ | ✓ |
| Parameters | ✓ | ✗ |
| Requires | - | Protocol-nfo text-file |
| Optional | Noise data | Noise data |
| Table 1. Overview of Siemens and Philips raw-export data and included and excluded attributes. |  |  |

### QOL Functionality

The user will be prompted for parameters if a protocol text-file is not supplied while loading Philips data. However, automatic detection through filenames is in place:

- The converter GUI will first search for a protocol text-file with the same name as the .data/.list files i.e., myData.list  myData.txt.

- An UI prompt will ask for required parameters if none is found: nucleus, field strength, receiver bandwidth, and echo time.

## Processing

Script(s) of interest: 	memri_Processing_Scripted.m

memri_Processing_GUI.m

## Visualization

Script(s) of interest: CSIgui.m

An under-development new light-weight viewing tool will be included in a future release. CSIgui is made compatible with the MeMRI-PTB data structure and can be downloaded from GitHub too.

CSIgui on GitHub: https://github.com/Sugocy/CSIgui

## Quantification

Script(s) of interest: 	memri_Quantification_GUI.m

memri_Quantification_Scripted.m

The quantification functionality is not included in the alpha-release. A customized version of OXSA will be implemented and included in the future releases.