# Quick Start Guide

A short and concise guide to enable converting, processing, quantifying and visualizing raw-export metabolic imaging data from both Siemens and Philips MR platforms in MATLAB.

Contents

Installation	1

Github in MATLAB	1

Framework Explained	2

Loading raw data	2

Processing	2

Visualization	3

Quantification	3

Data Converter (Load)	3

Process Data	3

# Installation

The MeMRI PTB can be downloaded from github or retrieved via the MATLAB environment. See below for the latter, assuming git is installed on your PC (see https://gitforwindows.org/).

Download location: https://github.com/Illuminate-MeMRI/Metabolic-MRI-Toolbox

## Github in MATLAB

Create an empty folder for the MeMRI PTB files using the “Current Folder” pane via right-click > New > Folder. Open the directory, after renaming it to your liking, right-click inside the white space and select “Source Control”. The window below will appear:

Select Git as “Source Control Integration” and enter the github path (https://github.com/Illuminate-MeMRI/Metabolic-MRI-Toolbox) in the “Repository path” edit bar. Click “retrieve” at the bottom right and wait until MATLAB has downloaded the required files.

# Framework Explained

<This needs to be moved to main help file>

The MeMRI PTB can be executed using solely scripts or via a graphical user interface (GUI) and it combines four functions into one package:

- Converting data

- Processing

- Visualization

- Quantification

The script-of-interest for each function, hard requirements plus a short description is given in the next paragraphs. All scripts-of-interest are found in the root-folder of MeMRI PTB. All

### Reading raw data

Script(s) of interest: 	memri_DataConverter_Scripted.m

memri_DataConverter_GUI.m

Convert the raw data into a MATLAB format that includes all necessary parameters to process the spectroscopy data.

Philips exports raw MR spectroscopy data in a list- & data- file format, but lacks and protocol information or parameters. and Siemens exports it in a dat-file format.

The script or GUI load both data sets into MATLAB memory and export is appropriately.,

### Processing

Script(s) of interest: 	memri_Processing_Scripted.m

memri_Processing_GUI.m

### Visualization

Script(s) of interest: CSIgui.m

A new light-weight viewing tool will be written in the very near future.

### Quantification

Script(s) of interest: 	memri_Quantification_GUI.m

memri_Quantification_Scripted.m

A customized version of OXSA.

# Data Converter (Load)

Data can be loaded using either the MeMRI DataConverter GUI or Scripted version. Both use the same underlaying functions to read and convert the raw data to a MATLAB friendly format. Exported data is stored in a struct containing the data

### Philips

Raw data from Philips platforms exported as list/data files is supported in combination with a text-file export of protocol information. If the parameter text file is not present, the user is prompted for some hard-requirement parameters, namely: nucleus, magnetic field strength, receiver bandwidth and echo-time.

### Siemens

The Siemens twix file contains all necessary information

### Noise Measurement

A seperate noise measurement can be added during the conversion process.

List/data also has noise data – if not presented will use that. Siemens standard CSI data does not have this and therefor uses voxel noise.

# Process Data